CAM-CODER

Working of CAM-CODER (Application of Mechatronics)

- Light from the optical lens assembly projects an image onto the charged coupled device (CCD)/ CMOS imager.

- The CCD is a photosensitive array which is charged by the light falling on it.

- The charge is then converted into a continuous analogue voltage when the CCD charged elements are scanned line by line.

- After the scan is completed, the CCD elements are reset to start the exposure process for the next video frame.

- Embedded within the CCD is an analogue-to-digital converter to produce a digital output for further processing by the camera processing block ready for data compression by the MPEG codec.

- The camera processing chip carries out such functions as ‘steady shot’, zoom and focus motor control and dig- ital picture effects.

- The MPEG-coded data are fed into a video buffer. Digitised Y/C data are also fed into the electronic viewfinder (EVF) for monitoring by the user. Stereo sound from audio microphones are A/D converted and the PCM audio data placed into an audio buffer.

- The MUX/DEMUX receives the compressed video and PCM audio streams from the corresponding buffers, packetises and multiplexes them into a standard MPEG-2 program stream (PS) to be stored in a PS buffer.

- Data in the PS buffer are then used to write on the recording medium which could be a DVD disc, an HDD or a magnetic tape.

- In the playback mode, the process is reversed and this is the reason for using an MPEG codec chip instead of just a coder and MUX/DEMUX instead of just a MUX.

- In the playback mode, data from the recording medium are demultiplexed and decompressed and fed into the EVF for display.